FIRE DOOR INFORMATION FOR RESIDENTS

Key Facts about the importance of your Flat Front Door

Fire Safety England Regulations 2022

Regulation 10

Checks of fire doors in buildings below 11 metres

The regulations do not replace the existing duty for a responsible person to put in place general fire precautions in any premises covered by the Fire Safety Order, regardless of the building’s height.

The Fire Safety Act 2021 has clarified that in any residential building which contains two or more sets of domestic premises are within the scope of the Fire Safety Order.

Responsible persons for residential buildings below 11 metres in height have a duty to put in place general fire precautions in these buildings, this duty includes making sure that all fire doors – including flat entrance doors – are capable of providing adequate protection.

Responsible persons are also required to provide residents in all residential buildings with two or more sets of domestic premises with information on fire doors.

Information that the responsible persons should give to residents.

The information given to residents should cover the importance of keeping doors closed, that doors and self-closing devices are not tampered with and that any faults or damage to doors should be raised immediately.

Residents will receive this information when they move into a multi-occupied residential building and then on an annual basis.

Your flat main entrance door is a fire door and is essential to the safety of yourself, your household, and your neighbours.

Why is a Fire Door so Important?

Your flat main front door is an integral part of the fire  compartmentation of your home and will resist fire, smoke and heat passing through it for a designated amount of time when closed (normally for a minimum of 30 minutes). Most fires in blocks of flats start in the flat itself and therefore having a fully working fire door in place helps to stop the fire from spreading to the rest of the building enabling the fire service to put out the fire more quickly.

It is a legal requirement for the main entrance door of a flat which opens onto communal areas shared by tenants to be a fire door.

Importantant Facts about your Flat Main Front Fire Door

- Your fire door has components that have specifically tested for your door type.

- A fire door closer is fitted to ensure that it fully closes the door unaided to the closed position.

- Your fire door must always be kept on the closed position when not in use. Never leave the door wedged open.

- Intumescent strips and cold smoke seals are fitted around the door or frame to create a seal between the frame and the door. They expand to prevent smoke and fire from passing through the door.

- Three fire-resistant hinges are fitted, and these should therefore not be replaced with a different type of hinge.

- Intumescent materials will also be fitted within the door’s components such as letterboxes, vison panels and spy holes.

- Any glass panels present must consist of fire rated glass to ensure that the door provides at least 30 minutes fire resistance.

Some Simple Checks

That you can carry out to ensure your flat front entrance door provides adequate protection.

- The door fully closes -fire doors need to close automatically. Open the door halfway, let it go and allow it to close on its own. It should close fully into the frame without sticking to the floor and frame and without excessive gaps between the door and the frame (no more than 8-10mm at bottom of door and 4mm for the remainder).

- Door frame condition – door frames must be rigidly attached to the wall and free from damage.

- Seals are in place – fire doors should be fitted with intumescent strips and cold smoke seals. Ensure that these are well attached within the grooves in the frame or door leaf, continuous around the frame and are not damaged or missing.

- Door closer works correctly – fire doors must have a door closer to ensure they close automatically. Make sure this is correctly attached and not damaged in any way.

- Overall door condition – the fire resistance of the door will be compromised if it becomes damaged in any way so regular visual checks should be conducted.

If your flat main entrance door is damaged or you notice a defect please notify your Property Manager immediately and they will be able to advise who is responsible for the repairs under the terms of the lease

How can I tell if my flat front door is a fire door?

The best way to check if your flat front door is a fire door it to look at the top edge of the door leaf. There will usually be a certification label showing the fire door’s safety rating. FD-30 is the most common. If there is not one present or you are still unsure, then please contact your Property Manager for further guidance and assistance.

You must not modify your flat front door in any way.

- Do not add any additional locks or ironmongery.

- Do not drill or cut holes into the door or remove seals.

- Do not disconnect or remove the door closer or any other components added to the door as this will compromise its effectiveness,

- Do not change your flat main entrance door without first obtaining the necessary approval to do so. You should contact your Property Manager in the first instance who will advise on what permissions are required. Please note a replacement flat front door must be a fully certified fire door.